Particle data treatment

The goal of this task is to load the datafile from each of the two aerosol instruments (NS and OPS) and perform analysis of the data.

- Load the NS files and combine them in one pandas Dataframe

- Load the OPS files in the same script and combine them into another Dataframe

- Combine the two datasets into one single Dataframe, where you discard the largest sizebin of the NS and readjust the second to largest sizebin, so it cuts at 300 nm. Remember to account for the reduced width of the sizebin by readjusting the particle number accordingly. This combination enables particle measurements from 10-300 nm (with the nanoscan) and from 300-10.000 nm with the OPS.

- Generate a figure with two subplots. Let the top plot display total concentration vs time and let the bottom plot be a heatmap of particle size vs time with color as the particle concentration.

- Make a function that enables you to mark parts of the dataset based on start and end times (allowing you to isolate data related to specific work activities)

- Create several activities by specifying start and end times

- Convert the concentrations to particle mass – assuming spherical particles with a density of 1 g/cm3

- Determine the average PM4 level for each activity over an 8-hour period, assuming exposure during the activity itself and exposure to a concentration of 0 #/cm3 for the remainder of the time (PM4 is summed particle mass of particles smaller than 4 µm equivalent to 4000 nm)